# MINI WEATHER MONITORING SYSTEM

Using NodeMCU ESP8266 | OLED Display | DHT11 Sensor | WiFi Web Server

# 1. Project Overview

This project demonstrates a fully functional Mini Weather Monitoring System using the NodeMCU ESP8266 microcontroller. The system measures Temperature and Humidity using a DHT11 sensor and displays the live readings on a 0.96 inch OLED screen.

The NodeMCU is also connected to WiFi and hosts a local web server, allowing users to monitor weather data from any smartphone or computer connected to the same network.

The OLED screen rotates between multiple pages such as weather data, current condition, and WiFi information, making the system interactive and user friendly.

This project combines embedded electronics, sensors, wireless communication, and display systems into one compact IoT device.

# 2. System Components

| Component | Specification | Qty | Purpose |
| --- | --- | --- | --- |
| NodeMCU ESP8266 | WiFi Microcontroller | 1 | Main controller |
| DHT11 Sensor | Temp + Humidity Sensor | 1 | Reads environmental data |
| OLED Display | 0.96 inch I2C SSD1306 | 1 | Displays readings |
| Jumper Wires | Male to Female | Several | Connections |
| Breadboard | Standard | 1 | Prototyping |
| USB Cable | Micro USB | 1 | Power + Programming |

# 3. Working Principle

The DHT11 sensor continuously measures surrounding temperature and humidity.

The NodeMCU reads this sensor data every few seconds and performs three operations:

- Shows data on OLED display

- Sends data to webpage using internal web server

- Evaluates comfort condition based on temperature and humidity

The OLED automatically changes pages after fixed intervals.

The webpage refreshes every 5 seconds to display updated values.

# 4. Circuit Connections

| NodeMCU Pin | Connected To | Function |
| --- | --- | --- |
| 3.3V | DHT11 VCC + OLED VCC | Power Supply |
| GND | DHT11 GND + OLED GND | Common Ground |
| D4 | DHT11 Data | Sensor Input |
| D2 | OLED SDA | I2C Data |
| D1 | OLED SCL | I2C Clock |

# 5. OLED Display Pages

Page 1: Weather Data

Displays:

- Temperature in °C

- Humidity in %

Page 2: Current Condition

Displays environmental status such as:

- Comfortable

- Hot

- Cold

- Humid

- Dry

Page 3: WiFi Status

Displays:

- Connected Network Name

- IP Address

- Signal Status

# 6. Web Server Output

When the NodeMCU connects to WiFi, it creates a local webpage accessible through browser.

Example: http://192.168.x.x

The webpage displays:

- Temperature

- Humidity

- Current Condition

- Auto refresh every 5 seconds

This allows remote viewing inside the local network.

# 7. Arduino Code Logic

Flow Sequence:

- Power ON system

- Show startup message on OLED

- Connect to WiFi

- Read DHT11 sensor

- Update OLED pages

- Start web server

- Wait for browser requests

- Repeat continuously

# 8. Technical Specifications

| Parameter | Value |
| --- | --- |
| Controller | ESP8266 NodeMCU |
| Input Voltage | 5V USB |
| Operating Voltage | 3.3V |
| Sensor Range Temp | 0°C to 50°C |
| Sensor Range Humidity | 20% to 90% |
| Display Type | OLED SSD1306 |
| Communication | WiFi 2.4 GHz |
| Refresh Rate | 5 sec |

# 9. Advantages

- Low-cost weather monitoring system

- Wireless data viewing

- Portable and compact

- Real time readings

- Low power consumption

- Easy to expand with more sensors

- Good beginner IoT project

# 10. Applications

- Home room monitoring

- Classroom environment checking

- Laboratory monitoring

- Greenhouse temperature watch

- Server room humidity check

- Smart home projects

# 11. Future Improvements

- Add Rain Sensor

- Add Air Quality Sensor

- Add Cloud Data Logging

- Mobile App Integration

- Public Internet Access

- Weather Alerts via Email

- Battery Backup System

# 12. Final Conclusion

This project successfully demonstrates how a NodeMCU ESP8266 can be used to build a smart weather station using IoT technology.

By combining sensors, OLED display, and WiFi communication, the system provides real time environmental monitoring in a compact and efficient design.

It is an excellent foundation for future smart city and smart home weather applications.